CORN UTILIZATIONS IN A FAMILY-BOILER: GREENHOUSE EMISSIONS, ECONOMIC ESTIMATION AND ENERGY ASSESSMENT

Gubiani R. 1, Dell’Antonia D. 1

1 University of Udine, The Department of Agriculture and Environmental Sciences – Division of Agricultural Engineering.

Key words: Corn, gas emission, boiler, economic, energy.

1. Introduction

The increased concentration of greenhouse gases observed in the atmosphere is changing the Earth’s climate.

Forests and agriculture play an important role in the global carbon cycle and have been given considerable attention as a way to reduce the accumulation of CO2 in the atmosphere. The paper addresses to three main objectives. The first is to quantify the amount of greenhouse gases during the combustion of corn and barley, as compared to the emissions value in a fossil combustion (natural gas, LPG and diesel). The second objective is to check economic convenience in a family run boiler using a corn to heat a house. The third is to estimated the net energy value of corn-combustion considering all the non-renewable energy required to grow corn.

2. Materials and methods

2.1. Greenhouse emissions

It was aimed to determine the air pollution (CO, NO, NO2, NOx, SO2 and CO2) from a corn-combustion in a family boiler for a heating. The flue gas emission was measured with “MRU vario plus”, which is a professional semi-continuous multi-gas analyzer device. The emissions of pollution gases were measured at a specific points on the stack, distant at least three time the diameter from curves and shrinkage. After that the boiler has been started, the emissions were recorded one each second, for a period of thirty minutes.

2.2. Economic estimation

The possibility of burning corn as fuel by using it in residential burners has been proposed to replace the traditional oil or gas fuel ones. Moreover, the possibility of burning corn allows to diversify the agricultural production towards the energetic compartment.

In order to verify the economic feasibility of a such possibility a comparison between traditional and corn burns has been made. In the first phase an economic model has been used for determining the utilization cost of corn boiler and its convenience in comparison to traditional boiler with using a diesel, LPG, wood and pellet fuels. Using an EXCEL sheet it was determined the thresholds of convenience of four system in a new installations and a change of already existing system. The economic analysis was carried out following a classic model “profit-cost”, for a boiler with 30 kW of power. In the second phase an economic model was used for determining the net present value (NPV) over ten years using as variable the price of corn and detecting the thresholds of economic convenience in a corn boiler with a 30 kW of power.

2.3. Energy assessment

Estimating the energy input for determining the net energy value of corn-combustion involves adding up all the non-renewable energy required to grow corn. In assessing inputs, the researchers considered such factors as the energy used in producing the crop (including production of pesticides and fertilizer, liquid fuels, natural gas, electricity, running farm machinery, irrigation, grinding and transporting the crop). It is crucial that the most current data available can be used to estimate the NEV of corn-combustion because the efficiency of growing corn and converting it to heat has improved significantly over the past 20 years.

3. Results

3.1. Greenhouse emissions

The results regard the emissions of corn, barley, wood, natural gas, LPG and diesel for a analysis mean of thirty minutes. Tree different models of corn-boiler have been use for an experiment and are indicated with A, B and C. The mean value of total pollutant concentration were converted in mg/m3 in a reference conditions of 11% to the residual oxygen, also for fossil fuel for being able to compare the values (tab. 1).

Table 1 - Total emissions of fuels.

| Boiler | Fuel | Oxygen | (CO) | (NO) | (NO2) | (NOx) | (SO2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (%) | (mg/m3) | (mg/m3) | (ppm) | (mg/m3) | (mg/m3) |
| Corn boiler A | Corn | 17 | 1,459 | 225 | 19 | 443 | 144 |
| Corn boiler A | Corn | 19 | 2,372 | 389 | 12 | 694 | 84 |
| Corn boiler A | Corn | 13 | 995 | N.D. | N.D. | 393 | 175 |
| Corn boiler B | Corn | 16 | 216 | 433 | 8 | 701 | 95 |
| Corn boiler C | Corn | 8 | 46 | 481 | 6 | 748 | 75 |
| Corn boiler A | Barley | 19 | 847 | 374 | 8 | 666 | 167 |
| Traditional stove | wood | 17 | 4,533 | 0 | 14 | 80 | 99 |
| Diesel boiler A | Diesel | 11 | 6 | 65 | 1 | 103 | 82 |
| Diesel boiler B | Diesel | 4 | 6 | 82 | 2 | 128 | 93 |
| LPG boiler A | LPG | 8 | 8 | 82 | 11 | 144 | 0 |
| LPG boiler B | LPG | 12 | 39 | 72 | 10 | 118 | 0 |
| Methane boiler A | Methane | 13 | 27 | 66 | 6 | 118 | 0 |
| Methane boiler B | Methane | 6 | 19 | 8 | 4 | 19 | 0 |

3.2. Economic estimation

In table 2 the profitable economic result are indicated with a green colour while the not convenient solution are indicated with a red colour. The blue colour points out the indifference economic solution (from -2,500 until +2,500). The result in the table show that in case of new installation of thermal system, burning corn is more convenient that diesel and LGP fuels. In case of replacing an already existing system with a corn boiler, convenience is limited to diesel and LPG fuels. Change wood and pellet boilers don’t result advantageous. In the second phase the NPV value over ten years shows that the not economic convenience in a corn boiler with a 30 kW of power if the price go over 0.27 euro for a low consumption, 0.39 euro for a medium consumption and 0.42 euro for a high consumption.

Table 2 – Comparison between corn system and traditional thermal system.

| Thermal system comparison with corn boiler | Consumption indices | New installation of thermal system | Change of already existing system with a corn boiler |
| --- | --- | --- | --- |
| Diesel | Low | 3,744 | 1,839 |
|  | Medium | 11,773 | 9,868 |
|  | High | 19,802 | 17,897 |
| LPG | Low | 9,787 | 7,883 |
|  | Medium | 23,861 | 21,956 |
|  | High | 37,934 | 36,029 |
| Pellet | Low | 2,839 | -3,352 |
|  | Medium | 5,677 | -513 |
|  | High | 8,516 | 2,325 |
| Log wood | Low | 6,618 | -8,144 |
|  | Medium | 4,664 | -10,098 |
|  | High | 2,711 | -12,051 |

3.3. Energy assessment

According to the energy coefficient by Pimentel, to produce average corn yield of 9.6 t/ha of corn using average production technology, requires the energy expense of about 9.7 million kcal for the large number of inputs. The total energy input to produce a kilogramme of maize is 1,008 kcal/kg, and the energy value of burning corn is 3,600 kcal/kg. Thus, in terms of energy output compared with the energy input for corn production, the study found that combustion of corn give a net energy profit of 2,592 kcal/kg (the energy ratio is 3.57). Not included in this analysis is the energy to transport the corn in the boiler for the combustion.